Chemistry Department

University of North Carolina at Charlotte

Physical Chemistry Laboratory I

Data Analysis Assignment

Learning Objectives:

- Create a data analysis worksheet to use for experiments in Chem 3141/Chem 3142

- Learn how to analyze and plot linear, polynomial, and general functions

- Report parameters and their uncertainties to appropriate significant figures.

- Demonstrate an understanding of error propagation and reporting of results with correct significant figures and uncertainty.

Submission of the Excel file

At the end of the activities, you should have several worksheets in your Excel file.  Delete the worksheets that were produced by the Regression tool. There should be four worksheets remaining, each with a plot of the given data and the regression line or curve.  Each worksheet should also have a table of the final regression parameters and their uncertainties all displaying the proper units and correct number of significant figures.  Make sure the parameters are clearly labeled as in the following example:

| b = | 0.623 | ± | 0.008 | unit |
| --- | --- | --- | --- | --- |
| m = | -3.884 | ± | 0.013 | unit |

When you have completed the workbook, save it as an Excel workbook with the name

yourlastname_ver_x.xlsm where x is the version number. Finally, upload the workbook to the course Canvas assignment.

5 pts will be deducted if file is not formatted and named correctly

You will be allowed 1 resubmission for a 10% deduction.

Exercises A-D will need to be approved before the Error propagation assignment  can be submitted.

## Exercise A: Calibration of a Pressure gauge using linear regression

Goal: Determine the calibration parameters with uncertainty for a pressure gauge using linear regression.

How do we know that a gauge is measuring the correct value?  Typically, a calibration is performed using an accurate or known value against the gauge.  In this activity you will watch the video of Exercise A and analyze the data collected.

Exercise A: Pressure Measurement

Reading: NGSK, Chapter 18 pp. 587-599

Objective:  The pressure of a sample of gas provides a relatively direct measure of the amount of gas contained. Pressures in the atmospheric range (0 – 1 atm) were once commonly measured using a mercury manometer, but this has mostly been replaced by various transducers that provide a mechanical or electronic response proportional to pressure such as a Bourdon gauge or a capacitance manometer. Our particular transducer is a capsule vacuum gauge which works by expansion and contraction of a thin-walled metal capsule that is translated into motion of a rotary dial. In order to calibrate this gauge, we’ll introduce various amounts of air into a vacuum manifold that is connected to both the capsule vacuum gauge and a closed-end mercury manometer. We’ll assume that the mercury manometer measures the true pressure in the manifold after a small temperature correction for the thermal expansion of mercury given by Equation 1.

Eq. 1		P(Torr) = P() (1 – 1.8x10-4 T) 		(T in °C)

We should find that the response of the capsule vacuum gauge is linear with pressure up to its limit of atmospheric pressure.

Experimental setup: Fig. 1 shows a diagram of the vacuum system. It includes a mechanical vacuum pump, a liquid nitrogen trap, a glass manifold, the capsule vacuum gauge, and the mercury manometer. The mechanical pump can reduce the pressure to ~100 mTorr which is low enough to assume that the pressure in the manifold is essentially zero when stopcock A is open. Stopcocks A, C, D, and E should initially be open and the others closed.

manifold

to vacuum

pump

Vacuum gauge

mercury manometer

A

E

D

B

capillary

C

Fig. 1: Vacuum system schematic

Procedure:

- Read the thermometer located near the manometer and record the temperature.

- Read the barometer to record the atmospheric pressure.

- Close stopcock E so that there’s zero pressure in the left arm of the manometer. Close stopcock A. Record the heights of mercury in the left and right arms of the manometer reading from the top of the menisci to the nearest mm in your lab notebook. Give the vacuum gauge a gentle tap with the pencil eraser and record the reading to 0.1 inHg.

- Looking at the vacuum gauge, open stopcock B (with two hands!) and allow about 1 inch of air to leak into the manifold. Record the left and right manometer readings and the vacuum gauge reading.

- Repeat step 4 until the gauge reading is about 1 inHg.

- Slowly open stopcock A to pump the air from the manifold.

Data Analysis A: Pressure Measurement

Linear Regression:

Most students are familiar with the “trendline” function in Excel charts. With a couple of mouse clicks, Excel can determine the equation of a line, for example

Eq. 2                       	y = mx+b

that represents the “best fit” to a linear data set and displays the y-intercept, b, and the slope m. The regression line is plotted on the same chart as the data. This is always necessary in order to judge the quality of the fit and the presumed linearity of the data.

What is missing is a measure of the uncertainties in these parameters. For this, we’ll use the Excel Regression tool following the steps below.

Download the special Excel file “solve_plus.xlsm” from the course Canvas site and open it to Exercise A.  This file includes sample data that you will be analyzing for all the activities.

As an example of a linear data set, we will use the data collected for calibration of a capsule vacuum gauge (Pgauge) vs. a mercury manometer (P). The experimental procedure  Where the mercury manometer is the known values.

- Sample data from the vacuum gauge readings are in column A and the mercury heights for the readings of the left and right are in columns B and C, respectively.  The barometric pressure is in cell B3 and the temperature of the manometer in cell B4.

- In column D, enter a formula that calculates Pgauge = Patm – Pdial (in inHg). In order to correct for the thermal expansion of mercury, enter a formula in column E that calculates the pressure P (in Torr) by subtracting the mercury heights in column B and C and using Equation 1.1 (Hint: Pay attention to units)

- Insert an XY scatter chart and plot Pgauge (x axis) vs. P (y axis). Use the  following specific format:

  - No gridlines, no legend, no title, inside x and y tic marks, x axis title 'xdata', y axis title (rotated) 'ydata', and data points as open markers, size 5, with no connecting lines.

  - Use Save as Template to store the chart format as “pchem” for later use. Then edit the axis labels to display the specific quantities plotted. Finally, expand the x and y axis limits so that the data points mostly fill the chart with no large empty space.

- Perform the linear regression using the Excel Regression tool.

  - Under the Data menu, select Data Analysis, and select Regression.

  - If the Data Analysis item is not present, click on the Excel (round) button, select Excel Options, select Add-Ins and check the Analysis ToolPak and (while you’re at it) Solver.

  - Choose the Regression tool and input the x and y values by selecting the appropriate cells and click OK. A new worksheet appears that shows the linear regression slope (X variable) and intercept along with their standard deviations (of the mean) Sm and Sb or Δm and Δb. These standard deviations are calculated with exactly the same formulas referred to in NGSK Chapter XXI, Eq. 42. (Note in the textbook format, slope is a1 and the y-intercept is ao.)

- Copy the linear regression parameters to some empty cells in the Sheet labeled Exercise A and label them clearly. Use Eq. 2 with the Pgauge (inHg) values and the linear regression parameters to calculate a column of values in column F. Label this column Pcalc. Right click on the chart and use Select Data to add the data series Pgauge (x), Pcalc (y). Display this series as a solid curve, no markers, with width 1 pt. This is the regression line and it should pass near all of the data points.

- On the worksheet, display the values of m and b and their standard deviations with labels and units. Format the individual cells so that the correct number of decimal places is displayed (Right click and select Format cells).

  - Use the convention that an uncertainty has one significant digit unless that digit is 1 or 2, in which case it has two significant digits. Then use the further convention: The last digit in the uncertainty is the last significant digit reported in the result.

Examples:       	b = 0.0234567 g, Δb = 0.000246 g  →   b = 0.02346 ± 0.00025 g

m = 1.6789x10-7 s, Δm = 0.03267x10-7 s → m = 1.68 ± 0.03 x10-7 s

Incorrect:		m= 1.6789x10-7 s, Δm = 0.03267x10-7 s  →  m≠ 168 ± 3 x10-5 s

For scientific notation, both value and uncertainty should be expressed to the same power of ten with the value (not the uncertainty) nearest to one. Be sure to report the proper units for each value.

## Activity B: Calibration of a thermistor

## using multiple regression

Goal: Determine the calibration parameters with uncertainty for a thermistor using multiple regression.

Similar to Exercise A, we can calibrate a thermistor which measures temperature, however, the relationship to the known temperature is not linear. In this activity you will watch the video of Exercise B and analyze the data collected with a nonlinear data set.

Exercise B: Temperature Measurement

Reading: NGSK, Chapter 17 pp. 571-574

Objective:  Various devices for measuring temperature are used in the physical chemistry laboratory. The most basic and direct is the mercury thermometer, the limitations of which are described in the NGSK lab textbook. But perhaps the most common temperature probe is the thermocouple, which is formed by fusing two dissimilar metal wires. A small potential difference develops across the junction that depends almost linearly on temperature. With a good voltmeter, temperature can be measured to 0.01 oC. For more precise temperature measurement a thermistor can be used. This device is typically a ceramic semiconductor whose resistance depends inversely on temperature.  Temperature can be measured to 0.001oC, but the response is highly nonlinear. It turns out that the inverse of the absolute temperature T is well described by a quadratic function of the natural logarithm of the resistance R divided by 1 Ω, as logs are unitless:

Eq. 3	1T=b+m1(ln R/Ω)+m2(ln R/Ω)2

where the coefficients b, m1, and m2 are determined by calibrating the thermistor at known temperatures.  In this lab, we will have a few experiments that will require you to fit your data to a nonlinear model.  This exercise will walk you through how to do so.

Data Analysis B: Temperature Measurement

Multiple Regression:

The linear regression formulas can actually be extended to any polynomial of order n:

Eq. 4 y=b+m1x+m2x2+m3x3...

The process is referred to as “multiple regression”.  Let us take the example of a quadratic function, n = 2. Three parameters will be needed to describe the data: b, m1, and m2.

- On Sheet 3, called Exercise B of your Excel file, the data collected for the calibration of a thermistor has been provided. We suppose that the inverse of temperature in Kelvin, 1/T, is a quadratic function of the natural logarithm of the thermistor resistance, R divided by the unit 1 Ω as shown in Equation 3.

  - Column A is the resistance and column B is the temperatures in oC

- Enter a formula that calculates ln(R/Ω) in column C, a formula that calculates (ln(R/W))2 in column D, and a formula that calculates 1/T (in inverse Kelvin) in column E.

- Plot 1/T (y axis) vs. ln(R/W) (x axis) as an XY scatter chart.  Under the Design menu (or right-click) select Change Chart Type, select Templates, and choose the 'pchem' template that was created in the linear regression exercise.  Edit the chart axis labels and expand the axes appropriately.

- Use the Regression tool as follows: Open the Regression tool. For the y data select column E, but for the x data select both columns C and D. Click OK.

  - From the new sheet of statistics that appears, copy the parameters and standard deviations for a0 (Intercept), a1 (X Variable 1), and a2 (X Variable 2) to an empty space on Sheet Exercise B.

- Use the ln(R/Ω) values in column C and the parameters b, m1, and m2 to calculate the corresponding 1/T values in column F using Eq. 3. Label this column 1/Tcalc. Plot the regression line on the chart by adding the series ln(R/Ω) (x values) vs. 1/Tcalc (y values) displayed as a solid curve, no markers, and width 1 pt.

- Display the parameters and their standard deviations on the worksheet with the proper units and correct number of significant digits.

Note: Any order polynomial can be fitted by creating more columns, i.e. xdata^3, xdata^4, etc. (from left to right) and selecting all of the columns as the x values in the Regression tool.

## Exercise C: Accounting for uncertainty in measured data using weighted linear regression

Goal: Determine the fitting parameters and their uncertainties for a linear data set with uncertainties in the dependent data.

In the first two activities, we have assumed that the random experimental errors in the measured ydata values are all of the same size.  For some experiments, however, the measurement uncertainties in some data values are larger than for others, in which case they should be given less weight in the statistical analysis. In this activity you will be provided data for the experiment: Kinetics of the decomposition of benzenediazonium ion.  No video or procedure is provided.

Exercise C: Weighted Linear Regression

For several experiments in the Chem 3141/3142 sequence, the measurement uncertainties in some data values are larger than for others, in which case they should be given less weight in the statistical analysis.  This is taken into account by assigning weighting factors, wi, in the equations for least squares analysis such as those for linear regression in NGSK Chapter XXI, Eqs. 20 – 21. Often the proper weighting factors are not known, but in the case where each ydata value is itself an average of several experimental measurements with a known standard deviation, the weight can be taken as the inverse of the square of the standard deviation. For data point i with y value        < ydatai > found by averaging mi replicate measurements:

Eq. 5               	wi = 1/si2  		     si2=1mi-1kmi(ydatai,k-<ydatai>)2

A weighted regression analysis thus requires N data pairs and N standard deviations. Unfortunately, the Excel Regression tool does not explicitly carry out weighted regressions, but it is possible to do weighted regressions if the data is entered in a special way as demonstrated in NGSK Chapter XXI Fig. 1.

- On Sheet 4 of your workbook, called Exercise C, you will find a data set associated with the kinetics of the decomposition of benzenediazonium ion. The object is to calculate the activation energy of the reaction by a linear regression of the Arrhenius equation expressed in logarithmic form:

Eq. 6               	ln k=-EaR1T+ln A

- Column A contains the temperatures at which the reaction was run, column B contains the rate constants measured at each temperature, and column C contains the standard deviations of the rate constants.

- Start by calculating 1/T (in inverse Kelvin) in column D and natural log of k (lnk) in column E. Calculate the uncertainties in the lnk values using the relation:

Eq. 7               	Δ(ln k)i = |Δk/ki|

in column F. Use the Excel function ABS() to calculate the absolute value.

- Plot lnk (y axis) vs. 1/T (x axis) and format the chart with the 'pchem' template saved earlier in the linear regression exercise.

  - Improve the appearance of the chart by adjusting the y axis to the range -10.5 to -5.5 with axis numbers showing just one decimal place and the x axis crossing at -10.5.

  - Format the x axis numbers so they show five decimal places.

  - Under the Layout menu (or Add Chart Element), select Error bars, then More Error bars options. For Error amount, click Custom. In the next dialog, select the column of uncertainties in column F for Positive Error Values and also for Negative Error Values. If your chart shows error bars in the x direction, click on one and hit the Delete key to leave only the y error bars.

- Now for the weighted regression.  In column G, calculate the square root of the weights wi1/2 according to Eq. 4: (wi1/2 = 1/Δ(lnk)i). They should be positive.

- In column H, calculate the product w1/2*1/T, and in column I, calculate w1/2  *lnk.

- Open the Data Analysis Regression tool and check the box “Constant is zero”. For the y range, select the w1/2 * lnk values in column I and for the x range select all of the values in columns G and H.  Click OK. The regression output will contain the intercept (X variable 1), the slope (X variable 2), and their standard deviations.

- In Column J, calculate lnkcalc values values using Eq. 6 with the 1/T values and the weighted regression parameters.

  - Plot the regression line on the chart as a solid line, no markers, with width 1 pt. Also click on the data points and add a linear trendline.  Note the difference between the two regression lines – the weighted regression line comes closer to the data points with smaller error bars. Remove the trendline and keep the regression line from the chart, as it was only added for comparison.

- On the worksheet, report the weighted linear regression parameters and their uncertainties with the proper units and number of significant digits.

## Exercise D: Calculation of bond length of a diatomic with uncertainties using generalize curve fitting

Goal: Determine the bond length (re), depth of potential energy curve (De) and the shape of the well curve (β) and their uncertainties using generalized curve fitting.

In this activity, you get an introduction to computational methods as a way to solve for molecular properties. You will use the Potential Energy file from Exercise 3 of the Gaussian activity.

Exercise D:  Calculation of the Potential Energy of a Diatomic Molecule using Gaussian

Reading:  NGSK, Chapter 3 pp 82-85.

Objective:  Computational methods are an increasingly valuable tool in all areas of chemistry. As computer speeds increase and computational algorithms become more accurate, it is possible to calculate molecular properties numerically such as the shapes and energies of molecular orbitals and charge distributions. With such information one can, say, estimate the stability of a proposed structure before attempting to synthesize it in the lab, or to calculate physical properties to see if they match those observed for an unknown substance. But one must also be wary of approximations in the computations that limit their accuracy. Computational methods range from lower level methods such as molecular mechanics and semi-empirical, to higher level ab-initio methods such as Hartree-Fock, density functional, and MP2. The higher level methods generally agree better with experimentally measured properties but require more time for the calculation, so one must often choose the most accurate computational method given the available time. In this course, we will use the program GaussView 6.1.1 throughout to compare calculated properties with our experimental results. This will inform us as to how much confidence we can place in a particular computational method.

In this exercise, we will calculate the electronic energy of an CO molecule as a function of the internuclear distance using density functional theory (DFT). You will fit the energy values to an analytical function known as the Morse potential:

Eq. 8		Vr=De1-exp-β(r-re) 2

where De is the depth of the potential energy curve, β determines its shape, and re is the internuclear distance at the energy minimum. This potential energy function governs the vibration of the molecule and comparison of the calculated and analytical curves will help determine whether the Morse potential accurately describes the vibrational spectrum of CO. In Experiment 11, you will measure De, β, and re experimentally.

Data Analysis D: Generalized Curve Fitting

Curve fitting for General Functions:

If the experimental data cannot be described by a polynomial (i.e. it is exponential, logarithmic, trigonometric, etc.), the Solver tool in Excel can be used.  Solver applies an iterative approach referred to as “generalized reduced gradient optimization” to determine the parameters in the function that best describes the data. Starting from an initial estimate of the parameters, the function is calculated and the differences between the experimental and calculated y values are squared and then summed. The parameters are incremented and the process is repeated until the sum, a quantity referred to as c2 (chi-squared), decreases to a minimum. This procedure is known as a “least squares” curve fit analysis.

As an example, we’ll consider the total energy (V) of a diatomic molecule as a function of internuclear distance r. The calculated energies should accurately describe the true energies of the molecule, but it would be extremely useful if there were a closed-form analytical expression for the molecular energy. We will attempt to model V(r) using the Morse potential function in Eqn 8 and employ Excel Solver to estimate the values of the three parameters De, β, and re that best match the Gaussian calculated energies. In this way we will be able to judge the suitability of Eq. 8 as the model for our actual molecule. A typical procedure using Solver to determine the parameters (or “fit the curve”) is outlined in the following where we treat the Gaussian calculated energies very much like we did the measured experimental data in the other exercises.

- On Sheet 5 of your sample workbook, labeled as Exercise D, the energies in kCal/mol and the internuclear distances in angstroms should have been added to columns A and C. Start by converting from kCal/mol to kJ/mol into Column B.

- Plotting the energies V (y axis) vs. r (x axis) and format the chart with the ‘pchem’ template created during the linear regression exercise.  Improve the appearance of the chart by entering axis labels with units (use A for the unit angstrom) and reformatting the axis labels so that the y axis numbers have no decimal places and the x axis numbers have one.

- One problem arises because Eq. 8 is zero at the minimum (r = re) whereas the Gaussian energies were reported with respect to the first energy computed. Let us adjust the Gaussian energies by placing the cursor on the data point closest to the minimum and reading the y value, then entering a formula in cell D9 that subtracts this y value from the numbers in column B so that the minimum energy becomes zero. Label this column Vdata. Now use the Select Data dialog to edit the data series to use the y values from column D instead of column B.

- In order to calculate the energies using Eq. 8, we need an estimate of the parameters.  Look at the plot and estimate roughly the r value at the minimum of the curve and enter this number in cell B5. Also look at the y value where the energy levels off and enter this value in cell B3. In cell B4, enter 1 (angstrom-1).

- Now use Eq. 8 with the distances in column C and the parameter values C3 –C5 to calculate the energies in column E. Label this column Vcalc. Add this series to the chart as a solid curve, no markers, width 1 pt. Note that the calculated curve passes near but not exactly over the "data" markers.

- In column F, enter a formula for the residuals squared: (Vcalc – Vdata)2 and in cell B6, enter a formula for c2 based on the sum of these values and the degrees of freedom 𝝼 = N – 3:

Eq. 9    𝞆2=𝛴(Vdatai-Vcalci)2v

Note that if Eq. 8 described the "data" exactly, each Vcalc would equal Vdata, column F would contain only zeroes, and 𝞆2 would be zero. But 𝞆2 will never be zero due to experimental error, so the best we can do is find the parameter values that give us the minimum 𝞆2.

- Choose Solver from the Data menu. Set the Target Cell (or Objective) to C6, Equal To to Min, and By Changing Cells to cells C3 through C5. If there is a box for “Make unconstrained variables non-negative”, make sure that it is unchecked. Click on Solve.  Click on Keep Solver Solution. Note that the values of De, b, and re have been replaced with new values. On the chart, the calculated curve should now coincide more closely with the data points if the curve fit was successful.

- Unfortunately, Excel does not have built-in functions to calculate the uncertainties in the Solver parameter values as it does for regressions. Consequently, a macro has been added to the spreadsheet file 'solve_plus' that calculates the appropriate uncertainties.

  - The equations used are best expressed in matrix notation and won’t be given here. To invoke the macro, type ^u (hold down the control key and type u). In the dialog box select in turn the Vcalc values (just click cell E9), the parameter values (B3 to B5), and the 𝞆2 value (B6). Click Calculate and the standard deviations in the parameters will be returned in the column to the right of the parameters.

  - Note: Mac users have run into problems with the macro and may need to use school computers to be able to run the program.

- Display the curve fit parameters and uncertainties on the worksheet using the proper units and correct number of significant digits.

Propagation of Error

Goal: Calculate results and propagate error using parameters and their uncertainties from Exercises A-D.  This activity will also be used to provide guidance on submission of error propagation for lab reports.

You must have a corrected spreadsheet before attempting these problems.

Submission instructions:

Submit the handout below along with  hand written calculations. You must show all work for the calculations requested in neat and organized handwritten submission. INCLUDING UNIT AND PROPER SIGNIFICANT FIGURES!  You will need to show all derivations of the expressions used. This activity will be graded separate from the other exercises and will be due 1 week after the spreadsheet has been approved.  Only 1 submission is accepted.

Points will be deducted if work is not legible or hard to follow or if submitted late.

Differential Error Analysis

The statistical analyses in the preceding exercises are usually just one step in determining the final results of a physical chemistry experiment. Typically, the final results are calculated from the statistical parameters and the final uncertainties are propagated from the statistical uncertainties using differential error analysis. In order to gain experience with “propagation of errors” type calculations, we will use the Excel results generated above to calculate related results.  Every experiment in Chem 3141 and Chem 3142 will require you to submit handwritten calculations which typically include error propagation.

In this assignment, an explicit equation is given for each of the four exercises above plus the values of any associated parameters. Your task is to calculate the indicated result and its uncertainty, the absolute standard deviation. The results and uncertainties should be written with the proper units and correct significant figures as described on p. 2.

Let’s go through how to calculate uncertainties and propagate them through two methods.  Then we will review reporting the values.

# Propagation of Error

When collecting data for an experiment, error and uncertainty will be introduced.  As you use the collected data for calculations, your final value needs to account for the uncertainties and errors from the experiment.  This portion of the activity will walk you through different formulas for accounting uncertainties and when to use them.

Let’s designate F to be the final result you calculated by measuring the variables x, y, and z.   We are assuming that the variables are mutually independent and their uncertainties are designated to be Δx, Δy, and Δz.  Mathematically, we can write these statements as:

F=ƒ(x,y,z,...)

If we review differential equations, we know that for infinitesimal changes in dx, dy, and dz will produce in F the infinitesimal change dF or

dF=∂F∂xdx+∂F∂ydy+∂F∂zdz

Or if the changes are finite but are still small enough that the values of the partial derivatives are not appreciably affected by the change we can represent dx using Δx. (Note:  This is equivalent to a Taylor expansion in which only the first power terms have been retained)  A more descriptive discussion and derivation of formulas can be found in the Garland/Nibler textbook in Chapter II page 52.  Propagation of random errors is the most common expression used for this course, the uncertainty of a calculated value  (dF) can be found using Equation 1, which assumes independent variables:

(1)

Equation 1 is referred to as the differential error analysis method or sometimes referred to as the rigorous method.

Take away message:

- Measured quantities (e.g. weights, volumes, temperature..etc.) have uncertainties which can be represented by dx or Δx.

- The uncertainty of the measured value is based on equipment, statistical data from repeated measurements,  and/or personal judgment.

- These uncertainties will impact the uncertainty of calculated values and thus must be accounted for.

- Partial derivatives are taken with respect to one variable at a time, assuming the other variables are held constant.  The derivative is then multiplied by the uncertainty of the variable and then squared.

- Each squared term represents the contribution of uncertainty of the variable towards the total uncertainty.

In cases where multiplication and division is used, we can assume that the variables are not codependent and use the relative (also called the absolute) error propagation.  You will see this formula used frequently in physical chemistry and in quantitative analysis courses as many equations we use are only multiplication/division.

The relative formula to compute the error is:

df=F*Δxx2+Δyy2...			(2)

Note:  Compare Equation 1 and 2.  What scenarios can you use equation 2 for?

### Largest source of error:

In both cases, you can determine the largest source of contribution of error by comparing the squared terms under the square root.  The larger the term, the larger the contribution.

Ex: If Δxx2>Δyy2 then the x parameter contributed to the error more than y

We can review how x was measured and propose methods of reducing the uncertainty.  Could be running more trials, improved instrumentation, etc.

### Reporting values with correct significant figures:

These are the same rules you will/have used in Quantitative analysis and Instrumental classes. Before we go through how to propagate uncertainties, how do we report them?

A measurement should be expressed to indicate its uncertainty. How?

Rule 1: An uncertainty Δ has one significant figure, unless it starts with 1 or 2 in which case it has two.

Example: An energy value and uncertainty was calculated to be:

E = 345.4512844321 J, ΔE = 0.01972480005 J

The uncertainty in E is ΔE=0.01972480005,

0.01972480005

First non-zero is a 1, must keep 2 sig figs

0.020

Rounding results in the following

Rule 2: The last digit in the uncertainty is the least significant figure 	that can be reported in the result.

Example: An energy value and uncertainty was calculated to be:

E = 345.4512844321 J, ΔE = 0.020 J

ΔE = 0.020 J

3 decimals

Answer must match

E = 345.451 J,

In your report, your value will look like:

E=345.451+ 0.020 J

Rule 3:The result and its uncertainty should be expressed to the same power, the value determines the power.

Example: 		t = 1.6789x10-6 s, Δt = 0.03267x10-7 s

The value of time, t, is raised to the power of -6, so you need report the uncertainty to the power of -6

Δt = 0.03267x10-7 s

Δt = 0.00000003267

Δt = 0.003267x10-6 s

Following rule 1 we get Δt = 0.003x10-6 s

Following rule 2 we have t = 1.679x10-6 + 0.003 x10-6 s   or

t= (1.679+ 0.003)x10-6 s

# Propagation of Error-Assignment

Name_________________		Section_______________

Attach your work to this page. Points will be deducted if work is not legible or hard to follow or if submitted late.

In Exercise A you calibrated a pressure gauge and found the linear equation with uncertainty in the y-intercept and slope.

- Using this function, what is the pressure if the pressure reading from the gauge (Pgauge) is 12 inHg?

- When finding the uncertainty for this equation, will you be using Equation 1 or 2?  Briefly defend your choice.

- Without plugging in any values, show the equation(s) and work of propagating the error assuming the uncertainty in Pgauge in Question 1 is zero.

- What is the error in P?  Include units

- Properly report the value of the pressure and its error with correct significant figures and units. Put a square around your final answer.

- Which variable was the largest contributor to the uncertainty in P? How would you improve uncertainty in this variable?

In Exercise B, you calibrate a thermistor using multiple regression fitting. You found the values and uncertainty in b, m1 and m2,

- Using these values calculate T in oC when the resistance (R) is 3 kΩ. Hints: Solve for 1/T first and pay attention to units.

- When finding the uncertainty for this equation, will you be using Equation 1 or 2?  Briefly defend your choice.

- Without plugging in any values, show the equation(s) and work of propagating the error assuming the uncertainty  in R in Question 7 is zero.  Hints: Solve for 1/T first.

- What is the error in T?  Pay attention to units!

- Properly report the value of the temperature and its error and put a square around your final answer.

- Which variable was the largest contributor to the uncertainty in T? How would you improve uncertainty in this variable?

In Exercise C, a weighted linear regression was completed where you found the values of the uncertainty in the y-intercept and slope, assuming uncertainty in y. Where the slope is equal to -Ea/R.

- Determine the value of the activation energy in units of kJ/mol.  Hint: don’t over think the equation.

- When finding the uncertainty for this equation, will you be using Equation 1 or 2?  Briefly defend your choice.

- Without plugging in any values, show the equation(s) and work of propagating the error.

- Using the uncertainty in the slope, what is the error in Ea?

- Properly report the value of the activation energy and its error and put a square around your final answer.

- Which variable was the largest contributor to the uncertainty in Ea? How would you improve uncertainty in this variable?

In Exercise D, you fit a curve to find the parameter values of the Morse Potential for carbon monoxide.

- Using these values calculate the potential energy (V(r))  in units of kJ/ mol when  the bond length (r) is exactly 1.5 Å.

- When finding the uncertainty for this equation, will you be using Equation 1 or 2?  Briefly defend your choice.

- Without plugging in any values, show the equation(s) and work of propagating the error assuming the uncertainty  in r in Question 19 is zero.  Hint:  For this one, you really want to break up the steps.

- What is the error in energy?  Pay attention to units!

- Properly report the value of the energy and its error and put a square around your final answer.

- Which variable was the largest contributor to the uncertainty in V(x)? How would you improve uncertainty in this variable?

References

1.    Experiments in Physical Chemistry, 9th Ed., Nibler, Garland, Stine, and Kim, McGraw-Hill, New York, 2014