Chemistry Department

University of North Carolina at Charlotte

Physical Chemistry Laboratory I

Data Analysis Assignment

Learning Objectives:

- Create a data analysis worksheet to use for experiments in Chem 3141/Chem 3142

- Learn how to analyze and plot linear, polynomial, and general functions

- Report parameters and their uncertainties to appropriate significant figures.

- Demonstrate an understanding of error propagation and reporting of results with correct significant figures and uncertainty.

Submission of the Excel file

At the end of the activities, you should have several worksheets in your Excel file.  Delete the worksheets that were produced by the Regression tool. There should be four worksheets remaining, each with a plot of the given data and the regression line or curve.  Each worksheet should also have a table of the final regression parameters and their uncertainties all displaying the proper units and correct number of significant figures.  Make sure the parameters are clearly labeled as in the following example:

| b = | 0.623 | ± | 0.008 | unit |
| --- | --- | --- | --- | --- |
| m = | -3.884 | ± | 0.013 | unit |

When you have completed the workbook, save it as an Excel workbook with the name

yourlastname_ver_x.xlsm where x is the version number. Finally, upload the workbook to the course Canvas assignment.

5 pts will be deducted if file is not formatted and named correctly

You will be allowed 1 resubmission for a 10% deduction.

Exercises A-D will need to be approved before the Error propagation assignment  can be submitted.

## Exercise A: Calibration of a Pressure gauge using linear regression

Goal: Determine the calibration parameters with uncertainty for a pressure gauge using linear regression.

How do we know that a gauge is measuring the correct value?  Typically, a calibration is performed using an accurate or known value against the gauge.  In this activity you will watch the video of Exercise A and analyze the data collected.

Exercise A: Pressure Measurement

Reading: NGSK, Chapter 18 pp. 587-599

Objective:  The pressure of a sample of gas provides a relatively direct measure of the amount of gas contained. Pressures in the atmospheric range (0 – 1 atm) were once commonly measured using a mercury manometer, but this has mostly been replaced by various transducers that provide a mechanical or electronic response proportional to pressure such as a Bourdon gauge or a capacitance manometer. Our particular transducer is a capsule vacuum gauge which works by expansion and contraction of a thin-walled metal capsule that is translated into motion of a rotary dial. In order to calibrate this gauge, we’ll introduce various amounts of air into a vacuum manifold that is connected to both the capsule vacuum gauge and a closed-end mercury manometer. We’ll assume that the mercury manometer measures the true pressure in the manifold after a small temperature correction for the thermal expansion of mercury given by Equation 1.

Eq. 1		P(Torr) = P() (1 – 1.8x10-4 T) 		(T in °C)

We should find that the response of the capsule vacuum gauge is linear with pressure up to its limit of atmospheric pressure.

Experimental setup: Fig. 1 shows a diagram of the vacuum system. It includes a mechanical vacuum pump, a liquid nitrogen trap, a glass manifold, the capsule vacuum gauge, and the mercury manometer. The mechanical pump can reduce the pressure to ~100 mTorr which is low enough to assume that the pressure in the manifold is essentially zero when stopcock A is open. Stopcocks A, C, D, and E should initially be open and the others closed.

manifold

to vacuum

pump

Vacuum gauge

mercury manometer

A

E

D

B

capillary

C

Fig. 1: Vacuum system schematic

Procedure:

- Read the thermometer located near the manometer and record the temperature.

- Read the barometer to record the atmospheric pressure.

- Close stopcock E so that there’s zero pressure in the left arm of the manometer. Close stopcock A. Record the heights of mercury in the left and right arms of the manometer reading from the top of the menisci to the nearest mm in your lab notebook. Give the vacuum gauge a gentle tap with the pencil eraser and record the reading to 0.1 inHg.

- Looking at the vacuum gauge, open stopcock B (with two hands!) and allow about 1 inch of air to leak into the manifold. Record the left and right manometer readings and the vacuum gauge reading.

- Repeat step 4 until the gauge reading is about 1 inHg.

- Slowly open stopcock A to pump the air from the manifold.

Data Analysis A: Pressure Measurement

Linear Regression:

Most students are familiar with the “trendline” function in Excel charts. With a couple of mouse clicks, Excel can determine the equation of a line, for example

Eq. 2                       	y = mx+b

that represents the “best fit” to a linear data set and displays the y-intercept, b, and the slope m. The regression line is plotted on the same chart as the data. This is always necessary in order to judge the quality of the fit and the presumed linearity of the data.

What is missing is a measure of the uncertainties in these parameters. For this, we’ll use the Excel Regression tool following the steps below.

Download the special Excel file “solve_plus.xlsm” from the course Canvas site and open it to Exercise A.  This file includes sample data that you will be analyzing for all the activities.

As an example of a linear data set, we will use the data collected for calibration of a capsule vacuum gauge (Pgauge) vs. a mercury manometer (P). The experimental procedure  Where the mercury manometer is the known values.

- Sample data from the vacuum gauge readings are in column A and the mercury heights for the readings of the left and right are in columns B and C, respectively.  The barometric pressure is in cell B3 and the temperature of the manometer in cell B4.

- In column D, enter a formula that calculates Pgauge = Patm – Pdial (in inHg). In order to correct for the thermal expansion of mercury, enter a formula in column E that calculates the pressure P (in Torr) by subtracting the mercury heights in column B and C and using Equation 1.1 (Hint: Pay attention to units)

- Insert an XY scatter chart and plot Pgauge (x axis) vs. P (y axis). Use the  following specific format:

  - No gridlines, no legend, no title, inside x and y tic marks, x axis title 'xdata', y axis title (rotated) 'ydata', and data points as open markers, size 5, with no connecting lines.

  - Use Save as Template to store the chart format as “pchem” for later use. Then edit the axis labels to display the specific quantities plotted. Finally, expand the x and y axis limits so that the data points mostly fill the chart with no large empty space.

- Perform the linear regression using the Excel Regression tool.

  - Under the Data menu, select Data Analysis, and select Regression.

  - If the Data Analysis item is not present, click on the Excel (round) button, select Excel Options, select Add-Ins and check the Analysis ToolPak and (while you’re at it) Solver.

  - Choose the Regression tool and input the x and y values by selecting the appropriate cells and click OK. A new worksheet appears that shows the linear regression slope (X variable) and intercept along with their standard deviations (of the mean) Sm and Sb or Δm and Δb. These standard deviations are calculated with exactly the same formulas referred to in NGSK Chapter XXI, Eq. 42. (Note in the textbook format, slope is a1 and the y-intercept is ao.)

- Copy the linear regression parameters to some empty cells in the Sheet labeled Exercise A and label them clearly. Use Eq. 2 with the Pgauge (inHg) values and the linear regression parameters to calculate a column of values in column F. Label this column Pcalc. Right click on the chart and use Select Data to add the data series Pgauge (x), Pcalc (y). Display this series as a solid curve, no markers, with width 1 pt. This is the regression line and it should pass near all of the data points.

- On the worksheet, display the values of m and b and their standard deviations with labels and units. Format the individual cells so that the correct number of decimal places is displayed (Right click and select Format cells).

  - Use the convention that an uncertainty has one significant digit unless that digit is 1 or 2, in which case it has two significant digits. Then use the further convention: The last digit in the uncertainty is the last significant digit reported in the result.

Examples:       	b = 0.0234567 g, Δb = 0.000246 g  →   b = 0.02346 ± 0.00025 g

m = 1.6789x10-7 s, Δm = 0.03267x10-7 s → m = 1.68 ± 0.03 x10-7 s

Incorrect:		m= 1.6789x10-7 s, Δm = 0.03267x10-7 s  →  m≠ 168 ± 3 x10-5 s

For scientific notation, both value and uncertainty should be expressed to the same power of ten with the value (not the uncertainty) nearest to one. Be sure to report the proper units for each value.